# RTL Project Description – UART Receiver (RTL Design and Verification Mini-Lab)

This project implements a Universal Asynchronous Receiver (UART RX) in Verilog. The design converts serial UART data (rxd) into parallel 8-bit output (m_axis_tdata) using oversampling, start-bit detection, and stop-bit validation. The project was developed as part of a reinforcement learning (RL) based take-home exercise, focusing on building and evaluating tasks that LLMs (Large Language Models) find challenging in hardware code comprehension and debugging.

## My Role

As the design and verification engineer, my responsibilities included:
• Analyzing the original UART RX Verilog design and understanding its finite-state machine (FSM) and oversampling logic.
• Designing three versions of the receiver — Easy, Medium, and Hard — by introducing controlled timing and sampling variations.
• Performing functional simulation using Icarus Verilog on EDA Playground and generating EPWave waveform logs.
• Verifying output data integrity by checking m_axis_tdata, m_axis_tvalid, busy, and frame_error signals.
• Documenting waveform analysis results for each difficulty version to demonstrate how early/late sampling affects frame capture.
• Organizing all code, testbench files, and waveforms into a reproducible codebase structure for submission.

## Key Learnings

• Understood the relationship between UART oversampling timing and bit-level data corruption.
• Learned to debug timing-related logic issues through waveform inspection.
• Developed systematic verification techniques for small RTL modules.
• Practiced reward shaping by structuring RTL tasks into Easy (100% pass), Medium (30–60% pass), and Hard (0% pass) tiers.